SMED-Based Digital Changeover Application for Productivity  Improvement

A dissertation submitted in partial fulfillment of the requirement for the award of a degree

in

Bachelor’s of Fashion Technology (Apparel Production)

Submitted By

## PALLAVI  YADAV

Under the Guidance of

## DR. ANKUR  SAXENA

## (Associate  Professor)

Department of Fashion Technology

National Institute of Fashion Technology, Jodhpur

April , 2026

## ABSTRACT

This study focuses on the implementation of the Single Minute Exchange of Dies (SMED) methodology to improve production efficiency in the garment manufacturing sector. The primary objective of the project is to reduce machine setup and changeover time by identifying and eliminating non-value-added activities, thereby enhancing productivity and operational performance.

The research was conducted in a garment production unit, specifically in the sewing section, where frequent style changes lead to significant downtime. A detailed analysis of the existing changeover process was carried out using time study, operator observation, and process mapping techniques. The study revealed inefficiencies such as excessive internal setup activities, lack of standardization, unnecessary motion, and absence of pre-preparation.

The SMED framework was implemented through systematic steps, including separation of internal and external activities, conversion of internal tasks into external ones, and streamlining of setup operations. Improvements such as tool organization, parallel operations, and standardized procedures were introduced to reduce setup time. As a result, the total changeover time was significantly reduced, leading to improved machine utilization, increased production output, and enhanced line efficiency.

The findings demonstrate that SMED is an effective lean tool for reducing downtime and increasing flexibility in garment manufacturing, especially in environments with high variability and frequent style changes. The study also highlights the importance of continuous improvement, worker training, and process standardization for sustaining efficiency gains. Furthermore, it suggests future scope for integrating digital systems, IoT, and data-driven decision-making to further optimize production processes.

CERTIFICATE

“This is to certify that this Project Report titled “SMED-BASED DIGITAL CHANGEOVER APPLICATION FOR PRODUCTIVITY IMPROVEMENT” is based on the original research work by Ms. PALLAVI YADAV, conducted under the guidance of  Mr. NAVEEN KUMAR towards partial fulfillment of the requirement for the award of the Bachelor’s Degree in Fashion Technology (Apparel Production), of the National Institute of Fashion Technology, Jodhpur.

No part of this work has been copied from any other source. Material, wherever borrowed, has been duly acknowledged.”

(Signature of guide)	(Signature of Student)

Pallavi  Yadav

Head IE                                                                                                                      NIFT Jodhpur

## ACKNOWLEDGEMENT

Nothing in this world can be achieved alone, continuous efforts and a determined and dedicated approach with supportive guidance is all that is required for the completion of a task. The teamwork of so many people for a certain amount of time with full determination led to this Graduation Project's fulfillment.

It is my distinct honour and indeed a great privilege to have worked under Dr. Ankur Saxena, Associate Professor, NIFT Jodhpur (Project Mentor). I thank him for his sustained guidance, critical appreciation and unfailing encouragement and support throughout the course of this Graduation  project.

Most dedicatedly, I convey my thanks to Dr. Puneet Kumar, NIFT Jodhpur for their support during my research.

I would like to extend my gratitude to Mr. Sameer Satupathy (Assistant Manager - Planning) for his valuable contribution in my graduation project work and to all the members of  Laguna clothing  for helping me out in the best possible ways during the period of my GP work. I would like to thank Mr. Sukund Kumar and Mr. Shivam Singh Yadav  for his dedicated help and support throughout the project.

A special thanks and sincere gratitude to my father Mr.Umesh Kumar Yadav (DM) for being the constant source of motivation and support not only in this project but throughout the life.

Table of Contents

CHAPTER-1

- INTRODUCTION	5

CHAPTER-2

REVIEW OF LITERATURE

- Lean Manufacturing in Garment Industry	6

- SMED Concept	7

- Previous  Studies…………………………………………………………………………………………………………………………………………………………………………………………………………………………8

- Research Gap………………………………………………………………………………………………………………………………………………………………………………………………………………….8

CHAPTER -3

SMED THEORY AND FRAMEWORK

- SMED…………………………………………………………………………………………………………………………………………………………………………………………………………………………….9

- Types of Set-Up……………………………………………………………………………………………………………………………………………………………………………………………………………..9

- SMED Stages………………………………………………………………………………………………………………………………………………………………………………………………………………..10

- SMED Phases……………………………………………………………………………………………………………………………………………………………………………………………………………….12

CHAPTER-4

METHODOLOGY

- Study Area	14

- Data Collection	14

- Current Data Analysis	15

- Tools used	16

- Implementation of SMED	16

CHAPTER -5

ANALYSIS OF CURRENT SYSTEM

- Existing Changeover Process	18

- Time Study	18

- Identification of waste	19

- Problem Idendified	19

CHAPTER -6

SMED  IMPLEMENTATION

- Seperation of activities (Internal vs External)	21

- Coversion Improvements……………………………………………………………………………………………………………………………………………………………………………………………….22

CHAPTER -7

RESULTS & ANALYSIS

- Before and After comparison…………………………………………………………………………………………………………………………………………………………………………………………24

- Productivity improvements……………………………………………………………………………………………………………………………………………………………………………………………24

- Efficiency Analysis…………………………………………………………………………………………………………………………………………………………………………………………………………25

CHAPTER -8

DISCUSSION OF FINDING

- Benefits of SMED Implementations…………………………………………..……………………………………………………………………………………………………………………………………26

- Challenges Faced During Implementations……………………………………………………………………………………………………………………………………………………………………27

CHAPTER-9

LIMITATIONS AND FUTURE SCOPE………………………………………………………………………………………………………………………………………………………………………………………28

CHAPTER -10

CONCLUSION………………………………………………………………………………………………………………………………………………………………………………………………………………………….30

CHAPTER-11

REFERENCES………………………………………………………………………………………………………………………………………………………………………………………………………………………..31

### CHAPTER 1

###### INTRODUCTION

India is one of the largest producers of textiles and garments and a major sourcing hub for readymade clothing. However, the industry faces challenges such as shorter fashion cycles, changing customer demands, smaller order quantities, and increasing global competition. These factors require manufacturers to adopt flexible and efficient production systems with quick response times.

To address this, garment units are implementing lean manufacturing techniques like 5S, Kaizen, Poka-Yoke, DMAIC, and especially SMED (Single Minute Exchange of Dies). SMED helps reduce setup and style changeover time, which minimizes “Start-up losses” and improves productivity, particularly for small batch production and complex designs.

SMED focuses on separating internal setup activities (done when machines are stopped) from external ones (done while machines are running), converting internal tasks into external, and eliminating non-value-added activities. Its goal is to reduce changeover time to less than 10 minutes.

The SMED process involves four main stages:

- Analyze current setup processes

- Separate internal and external activities

- Convert internal activities to external

- Simplify and standardize the setup process

By applying SMED, manufacturers can reduce downtime, improve efficiency, lower costs, and respond quickly to changing market demands, making production more lean and competitive.

### CHAPTER 2

###### REVIEW OF LITERATURE

## 2.1 Lean Manufacturing in Garment Industry

Lean manufacturing is a systematic approach focused on eliminating waste, improving productivity, and enhancing efficiency in production systems. In the garment industry, lean principles are widely adopted to manage high variability in styles, reduce lead time, and improve responsiveness to market demand.

Key lean tools used in garment manufacturing include the following:

- 5S (Sort, Set in Order, Shine, Standardize, Sustain): Ensures workplace organization, reduces unnecessary movement, and improves operational efficiency.

- Kaizen (Continuous Improvement): Encourages small, incremental improvements involving workers at all levels.

- Poka-Yoke (Error Proofing): Prevents defects by minimizing human errors during production.

These tools collectively enhance workflow efficiency, reduce non-value-added activities, and improve product quality.

## Figure 2.1: Lean Manufacturing Tools Relationship

## 2.2 SMED Concept

Definition:
SMED (Single Minute Exchange of Dies) is a lean tool developed to reduce setup and changeover time to less than 10 minutes by optimizing machine and process adjustments.

Importance in Manufacturing:
SMED plays a critical role in industries with frequent product changes, especially garments, where styles change rapidly. It enables:

- Faster changeovers between styles

- Reduction in machine downtime

- Increased production flexibility

- Better handling of small batch production

By minimizing start-up losses, SMED improves responsiveness to market demands and enhances overall efficiency.

| Parameter | Traditional Method | SMED Method |
| --- | --- | --- |
| Setup Time | 20–60 minutes | Less than 10 min |
| Machine Downtime | High | Low |
| Flexibility | Low | High |
| Batch Size | Large | Small |
| Productivity | Moderate | High |
| Waste | High | Reduced |

Table 2.2: Comparison of Traditional Setup and SMED

## 2.3 Previous Studies

Various studies across manufacturing sectors highlight the effectiveness of SMED and lean tools:

- In automotive and machining industries, SMED has significantly reduced setup time by separating internal and external activities.

- Research shows that converting internal tasks to external tasks improves machine utilization and reduces idle time.

- Studies also emphasize standardization and simplification of processes as key techniques for changeover reduction.

- Applications of SMED combined with lean tools like Kaizen and 5S have led to measurable improvements in productivity and cost reduction.

However, most studies focus on heavy manufacturing rather than labor-intensive sectors like garments.

## 2.4 Research Gap

Despite the proven benefits of SMED in various industries, its implementation in the garment sector is still limited.

- Garment manufacturing involves high variability in styles and frequent changeovers, yet systematic SMED adoption is low.

- Existing research lacks detailed studies on applying SMED specifically to sewing lines and apparel production.

- There is a need to explore practical SMED frameworks tailored for garment units to improve efficiency and reduce setup time.

This gap highlights the importance of studying SMED implementation in garment manufacturing to enhance competitiveness and responsiveness.

CHAPTER 3

SMED THEORY & FRAMEWORK

## 3.1 SMED

Single Minute Exchange of Dies (SMED) is a lean manufacturing technique developed by Shigeo Shingo to reduce setup or changeover time in production processes to less than ten minutes. The term “single minute” does not necessarily mean one minute, but rather any setup completed in a single-digit time frame (i.e., under 10 minutes).

SMED plays a critical role in modern manufacturing environments, especially in industries like textiles and garments, where frequent style changes, small batch production, and dynamic customer demands require high flexibility. By minimizing setup time, SMED enables manufacturers to switch between products quickly without significant downtime.

The primary goal of SMED is to reduce non-productive time, thereby increasing machine availability, improving production flow, and enhancing overall equipment efficiency. It supports lean principles by eliminating waste associated with waiting, unnecessary motion, and inefficient processes.

Implementation of SMED contributes to:

- Reduced production lead time

- Increased responsiveness to market changes

- Improved product quality and consistency

Thus, SMED is not just a time-reduction tool but a strategic approach that enhances competitiveness in fast-paced manufacturing sectors.

## 3.2 Types of Set-up

Setup activities in SMED are broadly classified into two categories:

### 1. Internal Set-up

### Internal setup refers to activities that can only be performed when the machine is completely stopped. These tasks directly contribute to production downtime and must be minimized or eliminated wherever possible.

Examples include:

- Changing dies, tools, or bobbins

- Adjusting machine settings

- Fixing components inside the machine

Internal setup is critical because it directly affects production efficiency. Excessive internal setup time leads to idle machines, delayed output, and increased operational costs.

### 2. External Set-up

External setup includes activities that can be performed while the machine is still running, without interrupting production.

Examples include:

- Preparing tools and materials in advance

- Pre-heating equipment

- Arranging components near the workstation

The key objective of SMED is to convert as many internal activities into external ones as possible, thereby significantly reducing machine stoppage time.

This distinction between internal and external setup forms the foundation of SMED implementation and is essential for achieving faster changeovers.

Setup Activities

¦

------------------------------------------------------

¦ ¦

Internal Setup External Setup

(Machine Stopped) (Machine Running)

Figure 3.1: Internal vs External Setup

## 3.3 SMED Stages

The implementation of SMED follows a systematic four-stage approach:

### Stage 1: Analysis of Current Setup Process

In this stage, the entire setup process is observed and documented in detail. Techniques such as time study, video recording, and process mapping are used to identify each step involved in changeover.

The objective is to:

- Understand existing inefficiencies

- Identify bottlenecks

- Measure current setup time

- This stage provides a baseline for improvement

### Stage 2: Separation of Internal and External Activities

Once the process is analyzed, all setup tasks are categorized into internal and external activities.

The goal is to:

- Clearly distinguish tasks that require machine stoppage

- Identify tasks that can be performed while the machine is running

This step is crucial because it lays the groundwork for reducing downtime.

### Stage 3: Conversion of Internal to External Activities

In this stage, efforts are made to convert internal setup tasks into external ones wherever possible.

For example:

- Pre-assembling tools before machine stoppage

- Using quick-release mechanisms

- Standardizing parts and procedures

This stage significantly reduces the time the machine remains idle and improves operational efficiency.

### Stage 4: Streamlining and Optimization

The final stage focuses on simplifying and improving all setup activities, both internal and external.

This involves:

- Eliminating unnecessary movements

- Using ergonomic tools and fixtures

- Implementing parallel operations

- Standardizing procedures

The aim is to achieve a fast, efficient, and repeatable setup process with minimal variation.

CHAPTER 4

METHODOLOGY

4.1 Study Area

The study was carried out in a garment production unit, focusing specifically on the sewing section where frequent style changes occur. The production line selected for analysis consists of multiple sewing machines arranged in a progressive bundle system, handling operations such as stitching, trimming, and finishing.

The chosen line produces shirts, where changeovers occur due to:

- Style variation

- Frequent non- standard operations

- Fabric types

- Thread color adjustments

This area was selected because it experiences high setup time losses, making it ideal for implementing Single Minute Exchange of Dies (SMED).

## 4.2 Data Collection

To understand the current inefficiencies, multiple data collection techniques were used:

### 1. Time Study

- A stopwatch method was used to record the time taken for each changeover activity

- Observations were repeated 5–7 times to ensure accuracy

- Average time values were calculated

### 2. Operator Observation

- Operators were closely monitored during changeover

- Delays due to manual adjustments, waiting time, and searching tools were noted

- Skill variations between operators were also recorded

## 4.3 Current Process Analysis

The existing changeover process was analyzed in detail by breaking it into individual steps.

### Typical Changeover Activities

- Machine stopping

- Removal of previous thread and attachments

- Needle replacement

- Thread reloading

- Machine cleaning

- Trial stitching and adjustments

| Activity | Time(Minutes) |
| --- | --- |
| Machine stoppage | 2 |
| Thread removal | 5 |
| Needle change | 4 |
| Thread setting | 6 |
| Machine cleaning | 5 |
| Trial & adjustment | 8 |
| Total Time | 30 minutes |

Table 4.1: Existing Changeover Time

### Key Problems Identified

- Most activities performed as internal setup (machine stopped)

- Lack of standard procedure

- Operators searching for tools

- No pre-preparation before changeover

- Excessive trial-and-error adjustments

## 4.4 Tools Used

To analyze and improve the process, the following tools were applied:

### 1. Stopwatch / Time Study

- Used for measuring precise time of each activity

- Helped identify bottlenecks

### 2. Process Charts

- Flow process charts were created to visualize:

  - Sequence of operations

  - Delays and redundancies

### 3. Work Sampling

- Random observations taken to identify:

  - Idle time

  - Non-value-added activities

### 4. Video Analysis

- Slow-motion review of recorded process

- Helped detect:

  - Unnecessary movements

  - Repeated adjustments

## 4.5 Implementation of SMED

The SMED methodology was applied in a step-by-step structured manner:

### Step 1: Separate Internal and External Activities

- Internal: Activities done when machine is stopped

- External: Activities done while machine is running

### Step 2: Convert Internal to External

- Tools, threads, and needles prepared in advance

- Machine cleaning partially done before stopping

### Step 3: Streamline Internal Activities

- Standardized procedures introduced

- Quick-change attachments used

- Operators trained for faster adjustments

### Step 4: Improve Efficiency

- Tool kits placed near machines

- Visual guides for thread setting

- Reduced trial time using pre-settings

CHAPTER 5

ANALYSIS OF CURRENT SYSTEM

## 5.1 Existing Changeover Process

### Time Study of Non-Standard Operations (Before SMED Implementation)

Total machines count : 1752 machines

## 5.2 Machines Details

## 5.3 Identification of Waste

### 1. Idle Time

- Operator spends 60 sec searching for bobbin

- Machine remains stopped unnecessarily

- Loss of productivity

### 2. Motion Waste

- Walking to storage (40 sec)

- Returning with bobbin (35 sec)

Total unnecessary movement = 75 sec

### 3. Waiting Waste

- Trial and adjustment delays (60 sec)

- Minor pauses between steps (~20 sec cumulative)

Total waiting waste ≈ 80 sec

### 4. Overprocessing

- Cleaning done inconsistently (sometimes repeated

## 5.4 Problems Identified

### 1. Delays  in  Changeover

- Searching tools/materials during machine stop

- No pre-preparation of bobbins

### 2. Inefficient  Setup  Process

- No fixed sequence followed

- Operator-dependent variation

### 3. Lack  of  Standardization

- No SOP (Standard Operating Procedure)

- Different operators follow different methods

### 4. Poor  Workplace  Organization

- No fixed location for bobbins/tools

- Absence of 5S implementation

### 5. No  External  Setup  Activities

- Everything done after stopping machine

- Huge opportunity loss for SMED improvement

CHAPTER  5

RESULT AND ANALYSIS

6.1  INTERNAL AND EXTERNAL ACTIVITIES – ANALYSIS AND RECOMMENDATIONS

| Activity | Current Category | Can be Converted? (Yes/No) | Proposed Category | Remarks |
| --- | --- | --- | --- | --- |
| Guide setup | Internal | Yes | External | Pre-set on standby machine |
| Thread matching | Internal | Yes | External | Thread kit prepared in advance |
| Tape roll loading | Internal | Yes | External | Keep tape ready before changeover |
| Machine cleaning | Internal | No | Internal | Essential before next style |
| Needle change | Internal | No | Internal | Safety-critical task |
| Folder alignment | Internal | Yes | Internal | Use fixed guide for faster setup |
| Trial stitching | Internal | No | Internal | Quality verification required |
| Tool collection | External | - | External | Done before machine stops |
| Temperature setting | Internal | No | Internal | Must be set on machine |
| Pressure setting | Internal | No | Internal | Dependent on fabric type |
| Fusing tape loading | Internal | Yes | External | Prepare tape roll beforehand |
| Machine cleaning | Internal | No | Internal | Required for bonding quality |
| Trial fusing check | Internal | No | Internal | Quality validation needed |
| Material preparation | External | - | External | Cut panels ready before changeover |
| Tool arrangement | External | - | External | Keep tools near workstation |
| Interlining matching | Internal | Yes | External | Issue correct interlining kit |
| Time/Speed setting | Internal | Yes | Internal | Use preset program if available |
| Patch template setup | Internal | Yes | External | Prepare template before changeover |
| Thread matching | Internal | Yes | External | Thread kit issued in advance |
| Bone patch alignment | Internal | No | Internal | Critical for accuracy |
| Machine cleaning | Internal | No | Internal | Required before next style |
| Needle change | Internal | No | Internal | Safety and fabric specific |
| Folder / guide setting | Internal | Yes | Internal | Use fixed guide/stoppers |
| Trial stitching | Internal | No | Internal | Quality check required |
| Tool arrangement | External | - | External | Prepare tool kit beforehand |
| Material fetching | External | - | External | Patch bundles ready before stop |
| Marker reference checking | Internal | Yes | External | Pre-marking done before setup |
| Guide setup (collar folder) | Internal | Yes | External | Pre-set folder on standby machine |
| Thread matching | Internal | Yes | External | Thread kit prepared beforehand |
| Lining bundle arrangement | Internal | Yes | External | Bundle prepared before changeover |
| Machine cleaning | Internal | No | Internal | Required for quality stitching |
| Needle change | Internal | No | Internal | Safety-critical |
| Folder alignment | Internal | Yes | Internal | Use fixed gauge for quick setting |
| Trial stitching on collar | Internal | No | Internal | Quality check mandatory |
| Tool preparation | External | - | External | Tools arranged in advance |
| Material fetching | External | - | External | Lining & collar parts ready |
| Guide/folder setup | Internal | Yes | External | Pre-set using duplicate folder |
| Thread matching | Internal | Yes | External | Thread kit issued before changeover |
| Collar part alignment check | Internal | No | Internal | Critical for quality |
| Machine cleaning | Internal | No | Internal | Required before next run |
| Needle change | Internal | No | Internal | Safety-critical task |
| Stitch length adjustment | Internal | Yes | Internal | Use preset dial marking |
| Trial run (sample stitching) | Internal | No | Internal | Quality verification |
| Tool preparation | External | - | External | Prepared in advance |
| Material (collar pieces) fetching | External | - | External | Kitting done before stop |
| Thread stand setup | External | - | External | Done prior to changeover |
| Guide setting (collar shape) | Internal | Yes | External | Pre-set guide on standby machine |
| Thread matching | Internal | Yes | External | Thread kit prepared beforehand |
| Dummy pattern alignment | Internal | No | Internal | Requires operator precision |
| Machine cleaning | Internal | No | Internal | Essential before stitching |
| Pressure foot adjustment | Internal | Yes | Internal | Use marked settings for quick adjust |
| Trial stitching | Internal | No | Internal | Quality check required |
| Tool preparation | External | - | External | Prepared before changeover |
| Collar bundle arrangement | External | - | External | Material staged in advance |
| Marking reference check | External | - | External | Done prior to machine stop |
| Band piece loading | Internal | Yes | External | Pre-arrange cut bands before changeover |
| Guide setting | Internal | Yes | External | Pre-set guide on standby machine |
| Thread matching | Internal | Yes | External | Thread kit prepared in advance |
| Machine cleaning | Internal | No | Internal | Must be done before stitching |
| Needle change | Internal | No | Internal | Safety and quality requirement |
| Folder adjustment | Internal | Yes | Internal | Use fixed folder position stopper |
| Trial stitching | Internal | No | Internal | Required for quality approval |
| Tool arrangement | External | - | External | Prepare tools before changeover |
| Material fetching | External | - | External | Ensure band bundles ready |
| Presser foot change | Internal | Yes | External | Keep alternate foot ready before stop |
| Thread color setup | Internal | Yes | External | Thread stand pre-loaded |
| Deco stitch program selection | Internal | No | Internal | Machine dependent setup |
| Fabric sample check | Internal | No | Internal | Required for accuracy |
| Guide marking adjustment | Internal | Yes | Internal | Introduce marked scale for quick setting |
| Bobbin winding | Internal | Yes | External | Pre-wound bobbins available |
| Machine lubrication check | Internal | No | Internal | Mandatory maintenance step |
| Tool arrangement | External | - | External | Organized tool tray used |
| Pattern sheet reference | External | - | External | Displayed near machine |
| Material alignment prep | External | - | External | Done before changeover |
| Spare needle availability | External | - | External | Kept in advance kit |
| Label checking | Internal | Yes | External | Labels verified before line stop |
| Thread replacement | Internal | Yes | External | Thread cone kept ready |
| Machine cleaning | Internal | No | Internal | Required for quality |
| Label positioning setup | Internal | No | Internal | Needs machine alignment |
| Guide adjustment | Internal | Yes | Internal | Introduce preset guide marks |
| Trial run | Internal | No | Internal | Mandatory for QC approval |
| Label bundle arrangement | External | - | External | Prepared by helper |
| Tool arrangement | External | - | External | Tool kit maintained at station |
| Work instruction review | External | - | External | Displayed SOP sheet |
| Loop folder setup | Internal | Yes | External | Pre-set folder on spare machine |
| Thread color change | Internal | Yes | External | Thread kit issued before stop |
| Loop cutting preparation | Internal | Yes | External | Pre-cut loops kept ready |
| Machine cleaning | Internal | No | Internal | Required for quality |
| Needle positioning | Internal | No | Internal | Precision task |
| Guide adjustment | Internal | Yes | Internal | Use marked settings for repeat styles |
| Trial run stitching | Internal | No | Internal | Essential for loop strength check |
| Tool arrangement | External | - | External | Organized before changeover |
| Loop bundle staging | External | - | External | Material issued in advance |
| Attachment marking check | External | No | External | Done during running |
| Air pressure check | External | No | External | Verified beforehand |
| Patch alignment marking | Internal | Yes | External | Marking template used before changeover |
| Thread shade verification | Internal | Yes | External | Thread card issued prior |
| Patch bundle arrangement | Internal | Yes | External | Pre-arranged in sequence |
| Presser foot adjustment | Internal | No | Internal | Required as per fabric thickness |
| Hoop/frame setting | Internal | Yes | Internal | Quick-lock mechanism can reduce time |
| Needle positioning | Internal | No | Internal | Precision required for patch curve |
| Trial run on scrap | Internal | No | Internal | Mandatory for quality approval |
| Tool preparation | External | - | External | Organized in tool kit |
| Material issue | External | - | External | Issued before machine stops |
| Machine oil check | External | - | External | Done during running condition |
| Workstation cleaning | External | - | External | Done parallelly |
| Button feeder setup | Internal | Yes | External | Pre-load feeder before changeover |
| Thread color change | Internal | Yes | External | Thread stand prepared in advance |
| Button size adjustment | Internal | No | Internal | Needs precision during setup |
| Folder/guide change | Internal | Yes | Internal | Introduce quick-lock mechanism |
| Machine oil check | Internal | No | Internal | Must be done before operation |
| Trial button stitching | Internal | No | Internal | Ensures button strength & alignment |
| Program setting (if semi-auto) | Internal | Yes | External | Pre-set program for next style |
| Button stock arrangement | External | - | External | Arrange buttons in kit form |
| Tool preparation | External | - | External | Keep tools ready at workstation |
| Sample verification | External | - | External | Approved sample kept ready |
| Cleaning workplace | External | - | External | Done before shutdown |
| Neck band folder setup | Internal | Yes | External | Pre-set folder on spare attachment |
| Top stitch thread change | Internal | Yes | External | Thread kit issued before stop |
| Fabric alignment check | Internal | No | Internal | Critical for seam accuracy |
| SPI (stitch per inch) adjustment | Internal | No | Internal | Quality dependent setting |
| Pressure foot adjustment | Internal | Yes | Internal | Use marked scale for faster setup |
| Edge guide positioning | Internal | Yes | External | Pre-mark guide positions |
| Trial run on sample | Internal | No | Internal | Mandatory for appearance check |
| Machine cleaning (lint removal) | Internal | No | Internal | Required for smooth stitching |
| Accessory staging (bands) | External | - | External | Prepared in bundles beforehand |
| Tool kit arrangement | External | - | External | Standard tool tray at station |
| Approved sample placement | External | - | External | Reference sample kept ready |
| Thread cone replacement | External | - | External | Done during running batch |
| Workstation inspection | External | - | External | Checked before changeover |
| Neck band folder setup | Internal | Yes | External | Pre-adjust folder on spare machine |
| Fabric panel alignment check | Internal | No | Internal | Critical for quality alignment |
| Thread replacement | Internal | Yes | External | Thread cone kept ready in advance |
| SPI & tension adjustment | Internal | No | Internal | Requires machine running condition |
| Guide plate fixing | Internal | Yes | Internal | Introduce marked positioning system |
| Machine cleaning | Internal | No | Internal | Basic maintenance required |
| Trial run with sample | Internal | No | Internal | Mandatory for seam accuracy |
| Operator instruction briefing | Internal | Yes | External | Explain changes before stop |
| Component bundling | External | - | External | Pre-bundled neck bands issued |
| Tool arrangement | External | - | External | Standard toolkit at station |
| Quality sample placement | External | - | External | Approved sample displayed |
| Line clearance | External | - | External | Done prior to style change |
| Tape width selection | Internal | Yes | External | Prepare tape roll as per spec beforehand |
| Measuring tape length | Internal | Yes | External | Pre-cut lengths using template |
| Tape cutting setup | Internal | No | Internal | Requires alignment for accuracy |
| Machine blade check | Internal | No | Internal | Safety and sharpness check mandatory |
| Initial sample cutting | Internal | No | Internal | Verification of correct size |
| Tape bundling | External | - | External | Bundle prepared before line feeding |
| Label marking on bundles | External | - | External | Color coding for easy identification |
| Material fetching | External | - | External | Pre-staged near workstation |
| Tool arrangement | External | - | External | Scissors, scale kept ready |
| Workstation cleaning | External | - | External | Done prior to changeover |
| Tape roll inspection | External | - | External | Check defects before use |
| Pleat marking alignment | Internal | Yes | External | Pre-mark panels before feeding |
| Folder setting | Internal | No | Internal | Requires precise adjustment on machine |
| Fabric panel arrangement | Internal | Yes | External | Bundle preparation before start |
| Thread replacement | Internal | Yes | External | Thread stand setup beforehand |
| Machine speed adjustment | Internal | No | Internal | Depends on fabric type |
| Trial pleat stitching | Internal | No | Internal | Quality check for symmetry |
| Guide positioning | Internal | Yes | Internal | Use reference scale for faster setup |
| Panel stacking | External | - | External | Stack in correct orientation |
| Tool readiness | External | - | External | Keep measuring scale & tools ready |
| Sample reference check | External | - | External | Approved pleat sample displayed |
| Workstation cleaning | External | - | External | Done during previous run |
| Extra fabric segregation | External | - | External | Avoid mixing styles |
| Collar panel alignment setup | Internal | No | Internal | Requires precision handling |
| Guide plate adjustment | Internal | Yes | Internal | Use calibrated marking for quick set |
| Thread tension setting | Internal | Yes | External | Pre-adjust thread tension unit |
| Fabric bundle positioning | Internal | Yes | External | Arrange bundles before stop |
| Needle position check | Internal | No | Internal | Critical for stitch accuracy |
| Trial seam matching | Internal | No | Internal | Visual quality verification needed |
| Marking reference alignment | Internal | Yes | External | Pre-mark collars before feeding |
| Pressure foot adjustment | Internal | Yes | Internal | Introduce preset pressure levels |
| Tool staging | External | - | External | Tools arranged beside machine |
| Bundle tagging verification | External | - | External | Done during previous operation |
| Workstation cleaning | External | - | External | Pre-clean before style change |
| Reference sample placement | External | - | External | Keep approved sample visible |
| Thread cone arrangement | External | - | External | Prepare thread stand in advance |
| Loop folder setup | Internal | Yes | External | Keep pre-adjusted folder ready |
| Thread change (loop matching) | Internal | Yes | External | Thread cone staged beforehand |
| Bartack program selection | Internal | Yes | External | Pre-load program for next style |
| Loop length adjustment | Internal | No | Internal | Requires precise measurement |
| Needle & bobbin check | Internal | No | Internal | Critical for stitch quality |
| Trial loop stitching | Internal | No | Internal | Mandatory for strength testing |
| Loop pieces arrangement | External | - | External | Prepared in bundles before stop |
| Tool positioning | External | - | External | Tools placed on workstation tray |
| Sample loop reference check | External | - | External | Approved sample kept ready |
| Workstation cleaning | External | - | External | Done during running style end |
| Tape folding setup | Internal | Yes | External | Pre-fold tape using manual jig |
| Hem guide adjustment | Internal | No | Internal | Requires precision alignment |
| Thread cone replacement | Internal | Yes | External | Thread stand pre-arranged |
| Looper threading | Internal | No | Internal | Needs machine stopped |
| Tape alignment trial | Internal | No | Internal | Critical for finish quality |
| Machine cleaning | Internal | Yes | External | Do cleaning during last batch |
| Tape roll inspection | External | - | External | Checked before shift |
| Tool kit positioning | External | - | External | Standardized tool tray |
| Material bundling | External | - | External | Cut pieces prepared in advance |
| Workstation layout check | External | - | External | 5S maintained before changeover |
| Piping roll preparation | Internal | Yes | External | Cut & prepare piping roll beforehand |
| Folder (piping foot) change | Internal | No | Internal | Requires precise fitting |
| Thread replacement | Internal | Yes | External | Thread stand arranged before stop |
| Needle positioning | Internal | No | Internal | Accuracy required for stitch quality |
| Machine speed adjustment | Internal | Yes | External | Pre-set standard RPM for style |
| Trial run on scrap | Internal | No | Internal | Ensures piping alignment |
| Fabric bundle staging | External | - | External | Bundle prepared near workstation |
| Tool kit arrangement | External | - | External | All tools placed in shadow board |
| Piping color verification | External | - | External | Color checked before start |
| Workstation cleaning | External | - | External | Done during last production run |
| Hem guide setting | Internal | Yes | External | Pre-adjust guide using reference sample |
| Piping alignment check | Internal | No | Internal | Critical for seam finish quality |
| Looper thread change | Internal | Yes | External | Pre-thread spare looper setup |
| Knife position adjustment | Internal | No | Internal | Requires machine stop & precision |
| Differential feed tuning | Internal | Yes | Internal | Standardize based on fabric type |
| Pressure foot calibration | Internal | No | Internal | Ensures smooth feeding of layers |
| Trial hemming with piping | Internal | No | Internal | Needed for visual and strength check |
| Stitch density adjustment | Internal | Yes | External | Pre-set SPI for style |
| Material bundle positioning | External | - | External | Organized near operator beforehand |
| Accessory (piping) staging | External | - | External | Sorted by color & size |
| Air blow cleaning | External | - | External | Done during previous run end |
| Bone patch alignment setup | Internal | No | Internal | Precision critical for placement |
| Clamp adjustment | Internal | Yes | Internal | Introduce preset clamp markings |
| Button program selection | Internal | Yes | External | Pre-load program for next style |
| Thread cone change | Internal | Yes | External | Thread kit prepared beforehand |
| Needle replacement | Internal | No | Internal | Safety & stitch quality requirement |
| Trial patch fixing | Internal | No | Internal | Required for strength verification |
| Sensor calibration (BT machine) | Internal | No | Internal | Machine-dependent accuracy step |
| Air pressure check | Internal | Yes | External | Pre-check compressor settings |
| Patch bundle arrangement | External | - | External | Bundle sorted by size beforehand |
| Tool positioning | External | - | External | Keep tools within arm’s reach |
| Lining piece alignment | Internal | No | Internal | Accuracy impacts final shape |
| Guide plate setup | Internal | Yes | External | Pre-set guide for next style |
| Thread tension setting | Internal | Yes | Internal | Mark standard tension levels |
| Needle check | Internal | No | Internal | Required for quality stitch |
| Sample run | Internal | No | Internal | Essential for alignment check |
| Fabric direction marking | Internal | Yes | External | Mark before machine stops |
| Bundle preparation | External | - | External | Lining pieces stacked in order |
| Tool arrangement | External | - | External | Use dedicated tool tray |
| Interlining check | External | - | External | Verify before operation |
| Cleaning area | External | - | External | Done parallelly |
| Looper thread change | Internal | Yes | External | Prepare thread cones in advance |
| Stitch density setting | Internal | Yes | External | Standardize SPI settings |
| Knife position adjustment | Internal | No | Internal | Precision critical |
| Pressure foot setting | Internal | Yes | Internal | Use reference markings |
| Trial stitch on scrap | Internal | No | Internal | Ensures seam strength |
| Collar piece staging | External | - | External | Prepared in sequence |
| Thread kit setup | External | - | External | Color-coded thread kit |
| Tool placement | External | - | External | Shadow board usage |
| Sample approval | External | - | External | Keep approved sample ready |
| Workstation cleaning | External | - | External | Done during last batch |
| Flatlock guide setting | Internal | No | Internal | Requires exact positioning |
| Thread path setup | Internal | Yes | External | Pre-thread machine partially |
| Differential feed adjustment | Internal | No | Internal | Fabric dependent |
| Speed calibration | Internal | Yes | External | Set standard RPM |
| Trial seam check | Internal | No | Internal | Check stretch & finish |
| Needle alignment | Internal | No | Internal | Critical for seam quality |
| Fabric layering setup | Internal | Yes | Internal | Use template for layering |
| Material staging | External | - | External | Prepared beforehand |
| Tool kit ready | External | - | External | Placed near operator |
| Quality sample check | External | - | External | Reference sample available |
| Tape roll replacement | Internal | Yes | External | Prepare roll in advance |
| Guide plate adjustment | Internal | No | Internal | Requires precision fitting |
| Thread setup | Internal | Yes | External | Thread stand pre-arranged |
| Needle check | Internal | No | Internal | Safety requirement |
| Tape alignment trial | Internal | No | Internal | Quality check step |
| Bundle staging | External | - | External | Material placed near machine |
| Tool arrangement | External | - | External | Shadow board system used |
| Tape width verification | External | - | External | Checked before changeover |
| Cleaning machine | External | - | External | Done during last batch |
| Helper support prep | External | - | External | Assistant prepares materials |
| Tack program selection | Internal | Yes | External | Pre-select program |
| Clamp positioning | Internal | No | Internal | Needs exact placement |
| Thread color change | Internal | Yes | External | Thread kit issued |
| Needle adjustment | Internal | No | Internal | Quality critical |
| Trial tacking | Internal | No | Internal | Strength testing |
| Air pressure setting | Internal | Yes | External | Pre-set compressor |
| Cuff bundle sorting | External | - | External | Prepared by helper |
| Tool setup | External | - | External | Tools kept ready |
| Sample approval check | External | - | External | Reference sample provided |
| Looper thread setup | Internal | Yes | External | Thread pre-loaded |
| Cuff edge guide setup | Internal | No | Internal | Needs accuracy |
| Differential feed adjustment | Internal | Yes | Internal | Use standard marking |
| Knife position check | Internal | No | Internal | Cutting accuracy required |
| Trial seam run | Internal | No | Internal | Ensures seam quality |
| Fabric bundle preparation | External | - | External | Sorted before start |
| Thread cone arrangement | External | - | External | Ready on standby |
| Tool placement | External | - | External | Organized in kit |
| Machine cleaning | External | - | External | Done before changeover |
| Operator briefing | External | - | External | Explain style differences |
| Button hole program selection | Internal | Yes | External | Pre-load design program |
| Knife size adjustment | Internal | No | Internal | Depends on fabric thickness |
| Thread setup | Internal | Yes | External | Thread stand prepared |
| Clamp alignment | Internal | No | Internal | Accuracy required for 12 holes |
| Machine calibration | Internal | No | Internal | Essential for uniform holes |
| Trial buttonhole stitching | Internal | No | Internal | Quality validation |
| Marking reference alignment | Internal | Yes | External | Pre-mark templates used |
| Needle check | Internal | No | Internal | Safety requirement |
| Speed setting adjustment | Internal | Yes | External | Standard speed preset |
| Cuff bundle staging | External | - | External | Bundle prepared beforehand |
| Tool arrangement | External | - | External | Tools placed near machine |
| Sample approval check | External | - | External | Approved sample ready |
| Temperature setting | Internal | No | Internal | Material dependent |
| Pressure adjustment | Internal | Yes | External | Standard preset chart |
| Fusing sheet placement | Internal | No | Internal | Needs precision |
| Timer setting | Internal | Yes | External | Pre-set cycle timing |
| Machine warm-up | Internal | No | Internal | Mandatory step |
| Trial fusing check | Internal | No | Internal | Bonding verification |
| Cuff panel arrangement | External | - | External | Prepared in advance |
| Fusing material staging | External | - | External | Sorted by size |
| Tool preparation | External | - | External | Ready before start |
| Cleaning belt surface | External | - | External | Done pre-changeover |
| Guide setting | Internal | Yes | External | Pre-set hemming guide |
| Thread change | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Essential |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Trial hem stitching | Internal | No | Internal | Ensures fold accuracy |
| Fabric bundle arrangement | External | - | External | Prepared earlier |
| Tool kit setup | External | - | External | Standard tool layout |
| Hem width marking | External | - | External | Use template marking |
| Workstation cleaning | External | - | External | Done before stop |
| Material flow setup | External | - | External | Line balancing done |
| Tape roll setup | Internal | Yes | External | Prepare tape roll beforehand |
| Folder alignment | Internal | No | Internal | Precision required |
| Thread setup | Internal | Yes | External | Thread prepared in advance |
| Needle positioning | Internal | No | Internal | Safety step |
| Machine speed tuning | Internal | Yes | External | Standard preset |
| Trial stitching | Internal | No | Internal | Check tape alignment |
| Cuff panel staging | External | - | External | Ready near machine |
| Tool kit arrangement | External | - | External | Shadow board system |
| Tape quality check | External | - | External | Pre-inspection |
| Cleaning workstation | External | - | External | Done earlier |
| Pattern guide setting | Internal | Yes | External | Use pre-marked guides |
| Thread color matching | Internal | Yes | External | Thread kit prepared |
| Looper adjustment | Internal | No | Internal | Machine-specific accuracy |
| Knife setting | Internal | Yes | Internal | Introduce stopper system |
| Trial stitching | Internal | No | Internal | Check pattern consistency |
| Speed synchronization | Internal | Yes | External | Preset machine speed |
| Fabric panel staging | External | - | External | Prepared in bundle form |
| Tool arrangement | External | - | External | Shadow board system |
| Pattern reference check | External | - | External | Sample kept ready |
| Workstation setup | External | - | External | Done before start |
| Material alignment | External | - | External | Pre-arranged panels |
| Program selection | Internal | Yes | External | Pre-load program |
| Knife adjustment | Internal | No | Internal | Fabric dependent |
| Thread setup | Internal | Yes | External | Thread ready beforehand |
| Clamp alignment | Internal | No | Internal | Accuracy needed |
| Trial stitching | Internal | No | Internal | Check hole quality |
| Needle inspection | Internal | No | Internal | Safety requirement |
| Speed setting | Internal | Yes | External | Standard preset |
| Bundle staging | External | - | External | Prepared in advance |
| Tool positioning | External | - | External | Nearby placement |
| Sample verification | External | - | External | Approved sample ready |
| Button feeder loading | Internal | Yes | External | Pre-load feeder |
| Program setting | Internal | Yes | External | Pre-select stitch pattern |
| Clamp positioning | Internal | No | Internal | Precision required |
| Thread replacement | Internal | Yes | External | Thread kit ready |
| Trial stitching | Internal | No | Internal | Check stitch strength |
| Sensor calibration | Internal | No | Internal | Machine dependent |
| Button arrangement | External | - | External | Sorted beforehand |
| Tool setup | External | - | External | Organized tools |
| Sample check | External | - | External | Approved sample ready |
| Workstation cleaning | External | - | External | Done earlier |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety requirement |
| Machine cleaning | Internal | No | Internal | Quality step |
| Trial stitching | Internal | No | Internal | Ensure dummy stitch look |
| Fabric staging | External | - | External | Prepared in bundles |
| Tool arrangement | External | - | External | Standard layout |
| Marking alignment | External | - | External | Use template |
| Workstation cleaning | External | - | External | Done earlier |
| Material flow setup | External | - | External | Pre-balanced line |
| Tape roll setup | Internal | Yes | External | Prepare tape roll beforehand |
| Split cuff alignment | Internal | No | Internal | Precision required |
| Guide setting | Internal | Yes | External | Pre-set guide system |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle positioning | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check tape alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Fabric staging | External | - | External | Prepared bundles |
| Tool kit setup | External | - | External | Shadow board |
| Tape inspection | External | - | External | Pre-check quality |
| Pattern setup | Internal | Yes | External | Pre-load stitch design |
| Thread color change | Internal | Yes | External | Prepare thread kit |
| Guide adjustment | Internal | No | Internal | Design accuracy needed |
| Needle change | Internal | No | Internal | Safety requirement |
| Trial stitching | Internal | No | Internal | Check design quality |
| Speed adjustment | Internal | Yes | External | Preset machine speed |
| Fabric staging | External | - | External | Prepared beforehand |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample ready |
| Workstation cleaning | External | - | External | Done earlier |
| Tape positioning | Internal | Yes | External | Pre-mark center line |
| Folder setup | Internal | No | Internal | Precision needed |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check center alignment |
| Panel staging | External | - | External | Prepared bundles |
| Tool kit setup | External | - | External | Standard layout |
| Tape inspection | External | - | External | Quality checked |
| Workstation prep | External | - | External | Done earlier |
| Material flow setup | External | - | External | Line balanced |
| Logo button loading | Internal | Yes | External | Pre-load feeder |
| Program selection | Internal | Yes | External | Pre-set program |
| Clamp alignment | Internal | No | Internal | Accuracy required |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Trial stitching | Internal | No | Internal | Check logo alignment |
| Sensor setup | Internal | No | Internal | Machine dependent |
| Needle check | Internal | No | Internal | Safety step |
| Button arrangement | External | - | External | Sorted beforehand |
| Tool setup | External | - | External | Organized tools |
| Sample verification | External | - | External | Approved sample ready |
| Temperature setting | Internal | No | Internal | Material dependent |
| Pressure setup | Internal | Yes | External | Use preset chart |
| Timer adjustment | Internal | Yes | External | Pre-set timing |
| Sheet placement | Internal | No | Internal | Precision needed |
| Trial bonding | Internal | No | Internal | Check adhesion |
| Panel staging | External | - | External | Prepared beforehand |
| Fusing material setup | External | - | External | Sorted materials |
| Tool arrangement | External | - | External | Ready tools |
| Machine cleaning | External | - | External | Done earlier |
| Workstation prep | External | - | External | Pre-changeover setup |
| Hem guide setting | Internal | Yes | External | Pre-set guide before stop |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle inspection | Internal | No | Internal | Safety requirement |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Trial hemming | Internal | No | Internal | Check fold accuracy |
| Fabric staging | External | - | External | Prepared bundles |
| Tool kit arrangement | External | - | External | Standard layout |
| Marking alignment | External | - | External | Use template |
| Material flow setup | External | - | External | Line balancing |
| Dart marking alignment | Internal | Yes | External | Pre-mark panels |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety requirement |
| Trial stitching | Internal | No | Internal | Check dart shape |
| Panel staging | External | - | External | Prepared earlier |
| Tool arrangement | External | - | External | Standard setup |
| Sample reference | External | - | External | Approved sample |
| Workstation cleaning | External | - | External | Done earlier |
| Material alignment | External | - | External | Bundle sorted |
| Pleat folding setup | Internal | Yes | External | Pre-fold panels |
| Guide setting | Internal | Yes | Internal | Use stopper system |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check pleat accuracy |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Panel staging | External | - | External | Prepared bundles |
| Tool setup | External | - | External | Ready tools |
| Sample check | External | - | External | Reference sample |
| Patch alignment | Internal | No | Internal | Precision critical |
| Guide setting | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed adjustment | Internal | Yes | External | Preset speed |
| Patch staging | External | - | External | Prepared earlier |
| Tool arrangement | External | - | External | Standard layout |
| Guide alignment | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle change | Internal | No | Internal | Safety requirement |
| Trial stitching | Internal | No | Internal | Check top stitch |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared bundles |
| Tool setup | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Hem guide setting | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety requirement |
| Machine cleaning | Internal | No | Internal | Quality step |
| Trial hemming | Internal | No | Internal | Check fold accuracy |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Shadow board |
| Marking alignment | External | - | External | Pre-mark panels |
| Workstation setup | External | - | External | Done earlier |
| Material flow setup | External | - | External | Line balanced |
| Looper thread setup | Internal | Yes | External | Thread prepared in advance |
| Knife adjustment | Internal | No | Internal | Fabric dependent |
| Guide setting | Internal | Yes | Internal | Add stopper system |
| Trial stitching | Internal | No | Internal | Check edge finish |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed adjustment | Internal | Yes | External | Preset speed |
| Gusset panel staging | External | - | External | Prepared bundles |
| Tool kit setup | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Loop strip cutting setup | Internal | Yes | External | Pre-cut strips |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Trial stitching | Internal | No | Internal | Check loop quality |
| Material staging | External | - | External | Prepared strips |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved loop sample |
| Workstation cleaning | External | - | External | Done earlier |
| Flow setup | External | - | External | Line balanced |
| Loop alignment | Internal | No | Internal | Precision required |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check loop position |
| Machine cleaning | Internal | No | Internal | Quality step |
| Loop staging | External | - | External | Prepared earlier |
| Tool setup | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Looper thread setup | Internal | Yes | External | Thread ready beforehand |
| Knife setting | Internal | No | Internal | Fabric dependent |
| Guide adjustment | Internal | Yes | Internal | Add stopper system |
| Trial stitching | Internal | No | Internal | Check seam finish |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed adjustment | Internal | Yes | External | Preset speed |
| Needle check | Internal | No | Internal | Safety step |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Tape roll setup | Internal | Yes | External | Prepare tape roll beforehand |
| Guide alignment | Internal | No | Internal | Precision required |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check tape alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Shadow board system |
| Tape inspection | External | - | External | Pre-check quality |
| Workstation prep | External | - | External | Done earlier |
| Pintak alignment setup | Internal | No | Internal | Precision required |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Guide setting | Internal | Yes | External | Pre-set guide |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check pintak consistency |
| Panel staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation cleaning | External | - | External | Done earlier |
| Material flow setup | External | - | External | Line balanced |
| Program setting | Internal | Yes | External | Pre-load program |
| Clamp alignment | Internal | No | Internal | Accuracy needed |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check bar tack strength |
| Sensor calibration | Internal | No | Internal | Machine dependent |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Sample check | External | - | External | Approved sample |
| Pintack guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check pintack design |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Yoke alignment setup | Internal | No | Internal | High precision needed |
| Guide setting | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check seam alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed adjustment | Internal | Yes | External | Preset speed |
| Fabric alignment | Internal | No | Internal | Double layer accuracy |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Check matching alignment | Internal | No | Internal | Pattern matching critical |
| Guide setting | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check check alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Edge guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle inspection | Internal | No | Internal | Safety requirement |
| Trial stitching | Internal | No | Internal | Check edge finish |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Marking alignment | External | - | External | Pre-mark darts |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Material flow setup | External | - | External | Line balanced |
| Program selection | Internal | Yes | External | Pre-load tack program |
| Clamp alignment | Internal | No | Internal | Accuracy required |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial tack | Internal | No | Internal | Check strength |
| Sensor setup | Internal | No | Internal | Machine dependent |
| Panel staging | External | - | External | Prepared gussets |
| Tool arrangement | External | - | External | Standard layout |
| Sample check | External | - | External | Approved sample |
| Hem + gusset alignment | Internal | No | Internal | Dual alignment needed |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Looper thread setup | Internal | Yes | External | Thread prepared |
| Knife adjustment | Internal | No | Internal | Fabric dependent |
| Trial stitching | Internal | No | Internal | Check seam quality |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed adjustment | Internal | Yes | External | Preset speed |
| Needle check | Internal | No | Internal | Safety step |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Label positioning setup | Internal | Yes | External | Pre-mark placement |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check label alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Label sorting | External | - | External | Sorted by size |
| Bundle staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Tape alignment setup | Internal | Yes | External | Pre-align tape strips |
| Yoke positioning | Internal | No | Internal | Accuracy critical |
| Looper threading | Internal | Yes | External | Pre-thread machine |
| Needle adjustment | Internal | No | Internal | Safety + stitch quality |
| Trial seam run | Internal | No | Internal | Check tape alignment |
| Machine calibration | Internal | No | Internal | Essential step |
| Panel staging | External | - | External | Prepared bundles |
| Tool kit setup | External | - | External | Organized layout |
| Tape inspection | External | - | External | Quality checked |
| Sample reference | External | - | External | Approved sample |
| Piping roll setup | Internal | Yes | External | Pre-cut piping rolls |
| Guide adjustment | Internal | No | Internal | Precision required |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle positioning | Internal | No | Internal | Safety step |
| Trial piping run | Internal | No | Internal | Check alignment |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Piping inspection | External | - | External | Quality check |
| Edge trimming setup | Internal | Yes | External | Pre-set trimming |
| Looper thread setup | Internal | Yes | External | Thread prepared |
| Knife adjustment | Internal | No | Internal | Fabric dependent |
| Trial finishing | Internal | No | Internal | Check seam finish |
| Machine cleaning | Internal | No | Internal | Quality step |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Material flow setup | External | - | External | Line balanced |
| Workstation prep | External | - | External | Done earlier |
| Fold alignment setup | Internal | No | Internal | Precision required |
| Guide setting | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check seam quality |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed setting | Internal | Yes | External | Preset speed |
| Fabric alignment | Internal | No | Internal | Double fold accuracy |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Second fold setup | Internal | No | Internal | Precision required |
| Guide adjustment | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check seam finish |
| Speed tuning | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Slit marking setup | Internal | Yes | External | Pre-mark slit lines |
| Guide positioning | Internal | No | Internal | Accuracy required |
| Thread setup | Internal | Yes | External | Thread kit prepared |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check slit finish |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Material alignment | External | - | External | Pre-arranged panels |
| Workstation prep | External | - | External | Done earlier |
| Slit hem alignment | Internal | No | Internal | Dual precision required |
| Looper threading | Internal | Yes | External | Pre-thread setup |
| Knife adjustment | Internal | No | Internal | Fabric dependent |
| Thread setup | Internal | Yes | External | Thread ready |
| Trial hem stitching | Internal | No | Internal | Check slit hem finish |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed tuning | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Label alignment setup | Internal | Yes | External | Pre-mark placement |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle inspection | Internal | No | Internal | Safety requirement |
| Trial stitching (4 sides) | Internal | No | Internal | Check label finish |
| Machine cleaning | Internal | No | Internal | Quality step |
| Speed setting | Internal | Yes | External | Preset speed |
| Label sorting | External | - | External | Sorted labels |
| Panel staging | External | - | External | Prepared bundles |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Label positioning | Internal | Yes | External | Pre-mark placement |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check alignment |
| Label sorting | External | - | External | Sorted by size |
| Panel staging | External | - | External | Prepared panels |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Material flow setup | External | - | External | Line balanced |
| Patch alignment | Internal | No | Internal | Precision required |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check patch finish |
| Speed setting | Internal | Yes | External | Preset speed |
| Patch staging | External | - | External | Prepared patches |
| Tool arrangement | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Looper threading | Internal | Yes | External | Pre-thread before stop |
| Knife setting | Internal | No | Internal | Fabric dependent |
| Yoke alignment | Internal | No | Internal | Accuracy needed |
| Thread change | Internal | Yes | External | Thread kit ready |
| Trial seam | Internal | No | Internal | Check seam finish |
| Machine cleaning | Internal | No | Internal | Quality requirement |
| Speed tuning | Internal | Yes | External | Preset RPM |
| Panel staging | External | - | External | Prepared panels |
| Tool layout | External | - | External | Standard layout |
| Sample check | External | - | External | Approved sample |
| Patch alignment | Internal | No | Internal | Shape precision needed |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check V shape |
| Patch staging | External | - | External | Prepared patches |
| Tool layout | External | - | External | Organized tools |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Manual alignment | Internal | No | Internal | Skill dependent |
| Guide setup | Internal | Yes | External | Pre-set guide |
| Thread setup | Internal | Yes | External | Thread kit ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check seam |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared panels |
| Tool layout | External | - | External | Standard layout |
| Sample check | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Material flow | External | - | External | Line balanced |
| Program setup | Internal | Yes | External | Pre-load program |
| Clamp alignment | Internal | No | Internal | Accuracy needed |
| Thread setup | Internal | Yes | External | Thread ready |
| Trial tack | Internal | No | Internal | Check strength |
| Sensor setup | Internal | No | Internal | Machine dependent |
| Speed setting | Internal | Yes | External | Preset speed |
| Panel staging | External | - | External | Prepared panels |
| Tool layout | External | - | External | Standard layout |
| Sample check | External | - | External | Approved sample |
| Label alignment | Internal | Yes | External | Pre-mark placement |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety step |
| Trial stitching | Internal | No | Internal | Check finish |
| Label sorting | External | - | External | Sorted labels |
| Panel staging | External | - | External | Prepared panels |
| Tool layout | External | - | External | Standard layout |
| Sample reference | External | - | External | Approved sample |
| Workstation prep | External | - | External | Done earlier |
| Material flow | External | - | External | Line balanced |
| Pleat alignment | Internal | No | Internal | Precision needed |
| Program setup | Internal | Yes | External | Pre-load |
| Thread setup | Internal | Yes | External | Thread ready |
| Trial tack | Internal | No | Internal | Check strength |
| Sensor setup | Internal | No | Internal | Machine dependent |
| Speed setting | Internal | Yes | External | Preset |
| Panel staging | External | - | External | Prepared |
| Tool layout | External | - | External | Standard |
| Sample check | External | - | External | Approved |
| Double layer alignment | Internal | No | Internal | Precision needed |
| Guide setup | Internal | Yes | External | Pre-set |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety |
| Trial stitching | Internal | No | Internal | Check seam |
| Speed tuning | Internal | Yes | External | Preset |
| Machine calibration | Internal | No | Internal | Essential |
| Panel staging | External | - | External | Prepared |
| Tool layout | External | - | External | Standard |
| Pleat folding | Internal | Yes | External | Pre-fold |
| Guide setup | Internal | Yes | External | Pre-set |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety |
| Trial stitching | Internal | No | Internal | Check pleat |
| Panel staging | External | - | External | Prepared |
| Tool layout | External | - | External | Standard |
| Sample check | External | - | External | Approved |
| Workstation prep | External | - | External | Done |
| Material flow | External | - | External | Balanced |
| Placket alignment | Internal | No | Internal | Precision needed |
| Guide setup | Internal | Yes | External | Pre-set |
| Thread setup | Internal | Yes | External | Thread ready |
| Needle check | Internal | No | Internal | Safety |
| Trial stitching | Internal | No | Internal | Check hem |
| Speed setting | Internal | Yes | External | Preset |
| Panel staging | External | - | External | Prepared |
| Tool layout | External | - | External | Standard |
| Sample reference | External | - | External | Approved |
| Workstation prep | External | - | External | Done |
| Neck alignment | Internal | No | Internal | Precision required |
| Guide setup | Internal | Yes | External | Pre-set guide |

6.2  BEFORE  AND  AFTER  SMED  IMPLEMENTATION

| BEFORE SMED (Total Time) |  |  |
| --- | --- | --- |
| Type | Total Time (sec) | Total Time (min) |
| Internal Time | 29,665 sec | 494.4 min |
| External Time | 18,865 sec | 314.4 min |
| Total Changeover Time | 48,530 sec | 808.8 min |

| AFTER SMED (Total Time) |  |  |
| --- | --- | --- |
| Type | Total Time (sec) | Total Time (min) |
| Internal Time | 17,920 sec | 298.7 min |
| External Time | 27,815 sec | 463.6 min |
| Total Changeover Time | 45,735 sec | 762.3 min |

| COMPARISON (Before vs After) |  |  |  |
| --- | --- | --- | --- |
| Parameter | Before SMED | After SMED | Change |
| Internal Time | 29,665 sec | 17,920 sec | 🔻 ↓ 39.6% |
| External Time | 18,865 sec | 27,815 sec | 🔺 ↑ 47.4% |
| Total Time | 48,530 sec | 45,735 sec | 🔻 ↓ 5.8% |

6.3  WEB  APPLICATION  FOR  DIGITAL  KANBAN

PUT  ALL  THE  UPDATED  AND  FINAL  POSSIBLE  SS  OF  WEB  APP  AND BELOW  THE IMAGE EXPLAIN  IT  PROPERLY .

6.4  CPM (COST  PER  MINUTE)

1. Given Data

| Parameter | Value |
| --- | --- |
| Total Budget per factory | ₹14,00,000 INR |
| Manpower | 160 |
| Working Days | 26 (standard). |
| Working Hours/Day | 9 hours |
| Efficiency (Initial) | 80%. |

## 2. Step-by-Step Calculations

### 🔹 Step 1: Total Available Minutes

Total Minutes  =160×26×9×60

=22,46,400 minutes

### 🔹 Step 2: Effective Minutes (at 80% efficiency)

Effective Minutes   =22,46,400×0.80

=17,97,120 minutes

### 🔹 Step 3: Initial CPM

CPM = 14,00,000/17,97,120

≈₹0.78 per minute

CPM Calculation After Efficiency Improvement (SMED Impact)

1. Given Data

| Parameter. | Value. |
| --- | --- |
| Total Budget | ₹14,00,000 |
| Manpower | 160 |
| Working Days | 26 |
| Working Hours/Day | 9 |
| Initial Efficiency | 80% |
| Efficiency Improvement | +11% |

## 2. New Efficiency

New  Efficiency  =  80% + 11% = 91%

## 3. Total Available Minutes

Total  Minutes  = 160 × 26 × 9 × 60

=22,46,400 minutes

# 4. BEFORE SMED (Initial Condition)

### 🔹 Effective Minutes

Effective Minutes = 22,46,400 × 0.80

= 17,97,120  minutes

### 🔹 Initial CPM

CPM  = 14,00,000/17,97,120

≈₹0.78 per minute

5. AFTER SMED (Improved Condition)

- Effective Minutes

Effective Minutes  =   22,46,400 × 0.91

= 20,44,224 minutes

- New CPM

CPM   = 14,00,000/20,44,224

≈₹0.68 per minute

6. Before vs After Comparison

| Parameter | Before SMED | After SMED |
| --- | --- | --- |
| Efficiency | 80% | 91% |
| Total Minutes | 22,46,400 | 22,46,400 |
| Effective Minutes | 17,97,120 | 20,44,224 |
| CPM (₹/min) | 0.78 | 0.68 |

## 7. CPM Reduction %

Reduction %  =  0.78  −  0.68/0.78×100

≈12.82%

CHAPTER 7

DISCUSSION OF FINDINGS

## 8.1 Observed Benefits of SMED Implementation

The implementation of the Single-Minute Exchange of Dies (SMED) technique in the garment production process has shown significant improvements in operational efficiency and productivity. Based on the analysis conducted throughout the project, the following benefits were observed.

### 1. Reduced Downtime

One of the primary advantages of SMED is the substantial reduction in machine downtime during style changeovers. By separating internal and external activities and streamlining setup procedures, the time required for changeovers was minimized. This led to better machine utilization and reduced idle time in the production line.

### 2. Increased Flexibility

SMED enabled the production system to respond quickly to changes in style and order requirements. The garment industry frequently deals with varying designs and small batch production; hence, improved flexibility allowed the unit to switch between styles efficiently without affecting productivity.

### 3. Faster Production Rate

With reduced setup time, more time was available for actual production. This resulted in an increase in the number of garments produced within the same time frame. Faster changeovers also ensured that production schedules were met more consistently.

### 4. Improved Workflow Standardization

The implementation encouraged the standardization of setup procedures. Clearly defined steps reduced confusion among workers and ensured consistency in operations across different shifts.

### 5. Better Resource Utilization

SMED reduced unnecessary movements, delays, and inefficiencies. This led to optimal use of manpower, machines, and materials, thereby improving overall operational performance.

## 8.2 Challenges Faced During Implementation

Despite the benefits, several challenges were encountered during the implementation of SMED in the garment production environment:

### 1. Worker Resistance to Change

Workers were initially resistant to adopting new methods and procedures. Since they were accustomed to traditional working styles, introducing standardized processes required time and continuous motivation.

### 2. Training  and  Skill  Development

SMED implementation required proper training of workers to understand new setup procedures and techniques. Conducting training sessions and ensuring that all workers followed the updated process was a significant challenge.

### 3. Initial  Time  Investment

In the early stages, analyzing processes, recording data, and redesigning workflows required additional time and effort. This temporarily affected production efficiency.

### 4. Coordination Between Departments

Effective SMED implementation required coordination between different departments such as cutting, sewing, and finishing. Lack of communication initially led to delays.

### 5. Maintaining  Consistency

Ensuring that standardized procedures were followed consistently over time was difficult. Continuous monitoring and supervision were required to sustain improvements.

CHAPTER 8

LIMITATIONS AND FUTURE SCOPE

## 9.1 Limitations of the Study

Despite achieving the objectives of analyzing and improving garment production efficiency using SMED principles and a web-based system, the study has certain limitations:

- Limited Data Scope
The study is based on a specific set of production data collected from a controlled or sample environment. It may not fully represent large-scale garment industries with diverse product lines.

- Time Constraints
Due to academic project timelines, long-term observation of improvements after SMED implementation was not possible.

- Simulation-Based Implementation
The SMED methodology was partially demonstrated through analysis and system design rather than full real-time industrial deployment.

- Dependency on Input Accuracy
The effectiveness of the web system depends on accurate data entry (running styles, upcoming styles, changeover time, etc.). Incorrect inputs may affect results.

- Limited Automation
The current system focuses more on monitoring and analysis rather than fully automated decision-making or machine integration.

- Technology Constraints
The web application (localhost-based) is not yet deployed on a live server, limiting accessibility and real-time industrial usability.

- User Adaptability
Workers and supervisors may require training to effectively use the system and understand SMED concepts.

- Lack of IoT/Real-Time Data Integration
The system does not currently integrate with sensors or machines for real-time tracking of production or changeover time.

## 9.2 Scope for Future Research

The project provides a strong foundation for further improvements and real-world implementation. Future enhancements can significantly increase its impact:

- Integration with IoT and Smart Machines
Connect machines with sensors to automatically capture changeover time, production speed, and downtime.

- AI-Based Optimization
Use Artificial Intelligence to predict optimal changeover strategies and reduce setup time further.

- Cloud-Based Deployment
Convert the localhost system into a cloud-based platform for access across multiple factories and locations.

- Advanced Data Analytics Dashboard
Add visual dashboards with graphs, KPIs, and performance insights for better decision-making.

- Mobile Application Development
Develop a mobile-friendly version for supervisors to track production on the shop floor.

- Automation of SMED Suggestions
Enable the system to automatically suggest improvements based on historical data and patterns.

- Scalability for Multiple Production Lines
Expand the system to handle multiple lines, departments, and factories simultaneously.

- Integration with ERP Systems
Link the application with existing ERP systems for seamless production planning and tracking.

- Sustainability Analysis
Extend the system to include carbon and water footprint analysis for eco-friendly garment production.

CHAPTER 9

CONCLUSION

This project focused on improving production efficiency using SMED principles and analyzing bobbin-related operations. The study identified inefficiencies in the setup process and reduced changeover time by converting internal activities into external ones.

Key outcomes include:

- Reduction in setup and idle time

- Improved machine utilization and workflow

- Optimization of bobbin handling and material flow

- Increase in overall productivity and efficiency

- Reduction in non-value-added activities

The implementation of SMED proved effective in enhancing operational performance with minimal resources. The project highlights the importance of continuous improvement, standardization, and operator involvement.

Further improvements can be achieved through regular monitoring and adoption of advanced technologies. Overall, the study demonstrates the practical benefits of lean manufacturing in improving productivity.

CHAPTER 10

REFERENCES

  Almomani, M.A. (2013). A proposed approach for setup time reduction through SMED. Journal of Manufacturing Systems.

  Mohammad, A. et al. (2023). Reduction of changeover time through SMED with RACI method. International Journal of Lean Six Sigma.

  Bajpai, J.D. (2014). SMED methodology in garment manufacturing industry: A case study.

  Jan, F. (2016). Application of SMED methodology in manufacturing industries.

  Dogan, O. (2018). Decision Support System for SMED implementation.

 Cakmakci, M. (2009). Process capability analysis using SMED methodology.

 Womack, J.P. & Jones, D.T. (2003). Lean Thinking. Simon & Schuster.

  Ohno, T. (1988). Toyota Production System: Beyond Large-Scale Production. CRC Press.

  Monden, Y. (1984). Toyota Production System. Industrial Engineering Press.

  Moxham, C. & Greatbanks, R. (2001). SMED application in textile processing industry.

  Groyyo Consulting (2022). SMED in apparel production: Agile manufacturing approach.